# Atlanticus R3.5
Performance / Stress Campaign Final Closure

Campaign Closure + R3.6 Entry Decision · v1.0.0 · 31 August 2026

| Campaign | R3.5 — Alarm Runtime Performance / Stress |
| --- | --- |
| Final status | CLOSED — PASS / GREEN |
| Final qualification | F-010 — CLOSED PASS/GREEN |
| Authoritative final run | 09311e68 |
| Qualified envelope | E2 = 1 CPU / 2 GiB |
| Qualified alarm load | 1000 integrated alarms |
| Iteration cadence | 5 s — qualified baseline |
| Physical data | Controlled physical bank; operational_representative=false |
| Open product findings | None |
| F-011 profiling | Not Activated / Not Required |
| Next campaign | R3.6 — Azure Deployment Qualification |
| First proposed increment | R3.6A-001 — Freeze Azure Runtime / Deployment Contract |

# 1. Executive closure

R3.5 is formally closed. The campaign established correctness, durability, temporal stability, physical-source execution, control-plane behavior, configuration adoption, constrained-container capacity and final integrated Docker qualification for the tested Alarm Runtime workload.

The final integrated qualification completed 361/361 iterations over 30 minutes at E2, with zero overruns, clean durability/materialization, exact Management + Decisions lifecycle, exact compatible configuration adoption and natural drain. No product defect remains open from the final campaign.

# 2. Frozen final operating conclusions

| Decision | Frozen conclusion |
| --- | --- |
| Recommended envelope | E2 = 1 CPU / 2 GiB |
| Minimum viable envelope | E2 = 1 CPU / 2 GiB |
| Capacity reference | F-007 E4 knee bracket [2000, 2500] alarms |
| Stress-reference load | 2000 alarms |
| Final integrated load | 1000 alarms |
| Iteration period | 5 s qualified baseline |
| Data refresh | 10 s remains campaign/harness publication cadence; not a universal external-source SLA |
| Integrity priority | Correctness/durability failures remain hard FAIL regardless of performance |
| Profiling policy | Profile only when a reproducible bottleneck exists |
| F-011 | Not Activated / Not Required |

# 3. Final F-010 evidence

| Metric | Authoritative result |
| --- | --- |
| Run | 09311e68 |
| Iterations | 361 / 361 |
| p50 / p95 / p99 | 3357.821 / 3500.548 / 4290.209 ms |
| Maximum iteration | 4902.872 ms |
| p95 start interval | 5001.632 ms |
| Overruns | 0 |
| Memory peak | 11.738% of 2 GiB |
| CPU throttling | 9.41652 s cumulative; no cadence loss |
| Durable records | 2121 |
| Journal | 19,980,659 B; aligned |
| Snapshots | 100 groups / 1000 alarms |
| Audit | PASS; 0 duplicate IDs, 0 chain mismatches, 0 materialization mismatches |
| Qualification | PASS/GREEN |

# 4. Integrated functional conclusion

| Dimension | Final result |
| --- | --- |
| Physical Latest / Historical | 1000 Latest + 2 DAILY; 60 min @ 10 s |
| Management requests / Decisions | 480 / 480 |
| ManagementEffect STARTED | 480 |
| DeactivationEffect STARTED / CLEARED | 480 / 480 |
| Lost / duplicate / correlation defects | 0 |
| Final pending backlog | 0 |
| Parameter adoption | PERF-AC-1 → PERF-AC-2; threshold 0.50 → 0.75 |
| Compatible changes | 1000 |
| Disabled / removed / reset / rejected | 0 / 0 / 0 / 0 |

# 5. Adaptive campaign disposition

R3.5 closes by evidence, not by mechanically executing every placeholder in the original 61-test plan. Superseded tests remain Superseded; deferred dimensions remain Deferred; conditional profiling remains Not Required unless future evidence activates it.

| Reserved test | Final disposition |
| --- | --- |
| F-008 | Deferred — not required for R3.5 closure |
| F-009 | Deferred — not required for R3.5 closure |
| F-010 | Closed — PASS/GREEN |
| F-011 | Conditional — Not Activated / Not Required |

# 6. Versioning decision

Campaign closure does not trigger a product package version bump by itself. The F-010 0.1.x corrective line belongs to qualification harness/audit behavior. Product packages retain their currently validated versions until a future product change requires an explicit increment.

R3.5 therefore closes as a campaign/release checkpoint. The next workstream is R3.6 at the campaign/architecture level; individual packages are incremented only when R3.6 produces an agreed code change.

# 7. R3.6 entry decision

Recommended next campaign: R3.6 — Azure Deployment Qualification.

R3.5 proved the runtime under local Docker limits chosen to be Azure-equivalent, but the physical bank remains controlled synthetic input and operational_representative=false. The next useful risk is therefore environment/deployment equivalence rather than additional local stress.

| R3.6A-001 contract item | Proposed frozen direction |
| --- | --- |
| Purpose | Define the real Azure execution/deployment contract before implementation. |
| Resource baseline | Start from E2 = 1 CPU / 2 GiB; no automatic resize. |
| Runtime cadence | 5 s baseline; preserve natural deadline/drain semantics. |
| Deployment target | Select the Azure container execution target explicitly by contract; no hidden platform assumptions. |
| Identity | Microsoft Entra ID where the runtime/deployment interaction requires identity. |
| Secrets | Key Vault / generic credentials only; never embedded in image, code or logs. |
| Storage / persistence | Name actual Azure bindings/access modes and preserve modular named connections. |
| Observability | Keep local enriched logs separate from Azure telemetry; remote export remains selective. |
| Evidence | Resource limits, startup, steady cadence, drain/restart and durable integrity on the selected Azure target. |
| Scope rule | Contract first. No Core/Persistence/runtime refactor until a real Azure gap is demonstrated. |

# 8. R3.6A-001 explicit exclusions

The entry increment does not redesign Alarm Core, add Redis, change Tool/UI contracts, introduce a new product observability framework, widen the F-007 dataset bank, or turn Atlanticus into an ADA-specific deployment monolith. Integrations are added only when required by the frozen Azure deployment contract.

# 9. Closure statement

R3.5 PERFORMANCE / STRESS CAMPAIGN: CLOSED / PASS-GREEN. Qualified recommendation: E2 = 1 CPU / 2 GiB with 5-second Alarm Runtime cadence for the tested workload. No open product finding. F-011 not required. Next: R3.6A-001 contract freeze only.